Notes on Outlandings

When to worry – Operating Band

25 to 40:1 gives lots of glide range

Three height bands

Upper – top third of the convective band, well above mountain peaks

Aggressive speed/thermal selection, moderate fallback concern

Mid – middle third of convective band – at mountain level

Balanced speed/thermal selection, increased consideration of fallback/gear change

Lower – below mountain level, ridge running

Racing less important/ recovering altitude or landing are the focus

The lower you go, the more you think about escape plans and fly slower

Work your way up to this – 20:1 bailout,

Tow altitude a good barometer – that’s how you started the day

Low Saves, and other Fairy Stories

Have rarely saved below 7-800 ft, let alone 500ft

Thermal is very, very narrow down low

Can only be partly centered even with 18 second circles, high g forces

Cores not yet developed, wander around

Wind turbulent low

This is very dangerous flying – so be realistic and give it up when the time comes

Time to Land

Completely normal procedure

Anxiety probably the most dangerous element, the stall/spin turn the executioner

Reasons not to worry

Slow touchdown speed/low momentum

No fire danger

Virtually all field landings survivable

As are tree landings

Precision flying – speed control is a must, the yaw string stays in the middle

Choosing fields – G’s Comments

Don’t just look under you

Don’t get too close as you approach

Look at the slope from a distance

SCOWL
	Slope

Drainage pattern

Furrows

Darker areas lower

Technique:  	Land up hill

Fly 15 knots faster

One wingtip down if necessary

Avoid cross slopes

Wings parallel, not level

Condition

Smooth/rough, hard/soft, wet/dry,

Cultivated or pasture?

Hayfield

Color – watch for color discontinuities

Crops – how high? – Prior observation important

Furrows – How deep, how recent, straight or undulating?

Stumps/brush

Holes

Animals – Cattle/Horses are curious, fields not smooth

Obstacles

Wires impossible to see

Must assume wires between all visible poles/buildings

Temporary fencing hard to see

Wind Direction and Strength

Strong Wind – steep approach

Low saves in strong wind highly dangerous!

Weak Wind – Shallow approach

Length of field

Practice from the air

Home runway

Football fields – athletic fields

Roadways about 25 ‘ wide

Telephone poles – varies but 200’ a good guess

Par 4 hole is 750 ft long minimum

Don’t change your mind on final

Ever!

Belts Tight

Speed control – speed kills – too much or too little

Fly a pattern

Gear always down

Stop short – no rollout

Flaps to negative on touchdown

Wingtip down, stick full back for groundloop if you must

Special Fun Landing Options

Water Landing

Wheel Down

Air Vents closed

Master switch off

Tree Landings

Minimum speed

Mush in

Danger climbing down