STANDARD  ECMA-180

PROTOCOL  FOR

COMPUTER-SUPPORTED

TELECOMMUNICATIONS  APPLICATIONS

(CSTA)

June 1992

Brief History

This Standard defines a Protocol for Computer-Supported Telecommunications Applications (CSTA) for OSI Layer 7 communication between a computing network and a telecommunications network. This Standard, plus its companion Standard ECMA-179 Services for Computer-Supported Telecommunications Applications, reflects agreements of ECMA member companies on the first phase of standards for CSTA. Additional phases are anticipated. This Standard is based on the practical experience of ECMA member companies and represents a pragmatic and widely based consensus.

This Standard takes its direction from Technical Report ECMA TR/52 Computer-Supported Telecommunications Applications.

Adopted as an ECMA Standard by the General Assembly of June 1992.

Table of contents

Page

µSection 1 - General	1

1	Scope	1

2	Conformance	1

2.1	     Static requirements	1

2.2	     Dynamic requirements	1

2.3	     PICS requirement	1

3	References	2

4	Definitions	2

Section 2 - Protocol Structure for CSTA	3

5	CSTA service definition model	3

5.1	     CSTA application layer structure	3

5.2	     Remote operations	3

5.3	     The CSTA service response	3

5.4	     Cross referencing of event reports	4

5.5	     Handling of private data	4

6	Interconnection service boundary	4

7	Security	4

Section 3 - CSTA Protocol	5

8	Association management	5

9	Switching function services	5

9.1	     Alternate call	6

9.2	     Answer call	7

9.3	     Call completion	8

9.4	     Clear call	9

9.5	     Clear connection	10

9.6	     Conference call	11

9.7	     Consultation call	12

9.8	     Divert call	13

9.9	     Hold call	14

9.10	  Make call	15

9.11	  Make Predictive call	16

9.12	  Query device	17

9.13	  Reconnect call	18

9.14	  Retrieve call	19

9.15	  Set feature	20

9.16	  Transfer call	21

10	Switching function events	22

10.1	     Call events	23

10.1.1	     Call cleared	23

10.1.2	     Conferenced	24

10.1.3	     Connection cleared	25

10.1.4	     Delivered	26

10.1.5	     Diverted	27

10.1.6	     Established	28

10.1.7	     Failed	29

10.1.8	     Held	30

10.1.9	     Network reached	31

10.1.10	     Originated	32

10.1.11	     Queued	33

10.1.12	     Retrieved	34

10.1.13	     Service initiated	35

10.1.14	     Transferred	36

10.2	     Feature events	37

10.2.1	     Call information	37

10.2.2	     Do not disturb	38

10.2.3	     Forwarding	39

10.2.4	     Message waiting	40

10.3	     Agent state events	41

10.3.1	     Logged on	41

10.3.2	     Logged off	42

10.3.3	     Not ready	43

10.3.4	     Ready	44

10.3.5	     Work not ready	45

10.3.6	     Work ready	46

10.4	     Maintenance events	47

10.4.1	     Back in service	47

10.4.2	     Out of service event	48

10.5	     Private events	48

11	Computing function services	49

11.1	     Route request	49

11.2	     Re-route request	50

11.3	     Route select request	51

11.4	     Route used request	52

11.5	     Route end request	53

12	Bidirectional services	54

12.1	     Escape service	54

12.2	     System status	55

13	Status reporting services	56

13.1	     Monitor start	56

13.2	     Change monitor filter	57

13.3	     Monitor stop	58

13.4	     Snapshot device	59

13.5	     Snapshot call	60

14	Switching function errors	61

15	Switching event cause values	63

16	CSTA data types	64

16.1	     Switching function objects	65

16.2	     Device identifiers	66

16.3	     Call and connection identifiers	68

16.4	     Connection states	69

16.5	     Status reporting	70

16.6	     Device and feature types and other parameters	73

16.7	     Security service	77

16.8	     Common extensions	78

Annex A - Protocol Implementation Conformance Statement (PICS) Proforma	79

# Section 1 - General

# 1	Scope

This Standard specifies application protocol data units (APDUs) for the services described in ECMA-179, Services for Computer-Supported Telecommunications Applications. The field of application of this Standard is the interconnection of switches and computers in a private telecommunications environment.

Section 2 (clause 5 to clause 7 inclusive) describes the concepts underlying the Remote Operations model, notation and service. Section 3 (clause 8 to clause 16 inclusive) contains CSTA-specific protocol details and forms the main body of this Standard.

The protocol defined in this Standard operates in the context of an application association. It is assumed that such an application association exists via mechanisms that are not defined in this Standard.

# 2	Conformance

A manufacturer may select any part (one or more operations) of the CSTA Protocol, as specified in this Standard, for implemention on a system.

A system is in conformance with one or more of the CSTA operations if those operations are implemented according to the definitions in this Standard.

A Protocol Implementation Conformance Statement (PICS) shall be used to specify the operations which are provided by a particular implementation. A PICS shall also specify the parameter options which are used.

A system claiming to implement one or more operations specified within this Standard shall comply with the relevant requirements in clauses 5 to 16 inclusive.

## 2.1	Static requirements

A system claiming conformance shall support the transfer syntax (derived from the encoding rules specified in CCITT Rec. X.209) named {joint-iso-ccitt(2) asn1(1) basic-encoding(1)}; for the purpose of generating and interpreting CSTA protocol information as defined by the abstract syntax "CSTA-ASN.1-Object-Descriptor" for the operations supported.

## 2.2	Dynamic requirements

A system claiming conformance shall:

i)	follow the procedures as specified in this Standard, and in ECMA-179, relevant to each CSTA operation that the system claims to implement, and

ii)	satisfy the definitions, as specified in ECMA-179, relevant to each CSTA service that the system claims to implement.

## 2.3	PICS requirement

The following shall be stated by the implementer when defining a PICS corresponding to an application or implementation:

i)	which CSTA operations, as defined in ECMA-179, are supported by the system for the particular implementation,

ii)	which optional parameters are supported by the PDUs belonging to the supported operations, and

iii)	the types and ranges of values for all the parameters supported.

A PICS Proforma is given in annex A.

# 3	References

ECMA-138	Security in Open Systems - Data Elements and Service Definitions (1989)

ECMA-155	Addressing in Private Telecommunication Networks (1991)

ECMA-179	Services for Computer-Supported Telecommunications Applications (CSTA) (1992)

ENV 41007	Definitions of Terms in Private Telecommunications Networks

ISO 9545	Open Systems Interconnection - Application Layer Structure

CCITT X.208 / ISO 8824	Specification of abstract syntax notation

CCITT X.209 / ISO 8825	Specification of basic encoding rules for the abstract syntax notation

CCITT X.217 / ISO 8649	Association control service definition

CCITT X.219 / ISO 9072-1	Remote operations - Part 1: model notation and service definition

CCITT X.227 / ISO 8650	Association control protocol specification

CCITT X.229 / ISO 9072-2	Remote operations - Part 2: protocol specification

CCITT E.164	Numbering plan for ISDN

All the CCITT Recommendations referenced above are the 1988 (Blue Book) versions.

# 4	Definitions

CSTA-specific terminology is defined in ECMA-179. For the purposes of this Standard, the following additional definitions, defined elsewhere, shall apply:

Remote Operations	as per CCITT Rec. X.219

Application Association	as per CCITT Rec. X.217

Application Context	as per CCITT Rec. X.217

Private Telecommunications Network	as per CENELEC ENV 41007

# Section 2 - Protocol Structure for CSTA

# 5	CSTA service definition model

## 5.1	CSTA application layer structure

The CSTA application layer structure conforms to the model as described in ISO 9545.

## 5.2	Remote operations

The services of CSTA are modelled as Remote Operations as described in CCITT Rec. X.219. Typically, one entity requests that a particular operation be performed; the other entity attempts to perform the operation and responds to the requestor. Consequently the operation of the protocol is an elementary request/reply interaction, supported within the OSI application layer, and carried out within the context of an application-association.

For some of the CSTA services, the entity to which the request is sent generates a reply which can indicate success or failure. For these services, CSTA shall adopt the Operations Class 2, defined in Rec. X.219 as:

-	Asynchronous, reporting success or failure (result or error).

For some of the CSTA services, the entity to which the request is sent generates a reply which can only indicate failure. For these services, CSTA shall adopt the Operations Class 3, defined in Rec. X.219 as:

-	Asynchronous, reporting failure (error) only, if any.

For some of the CSTA services, particularly the ongoing reporting of events, no reply is generated. For these services, CSTA shall adopt the Operations Class 5, defined in Rec. X.219 as:

-	Asynchronous, outcome not reported.

The protocol description for the particular service defines the relevant class of the Operation used for that service.

CSTA shall correlate the single response, denoting success or failure, with the originating request by using the mechanisms within the ROSE protocol.

## 5.3	The CSTA service response

CSTA employs a generic response mechanism which is, in principle, decoupled from the specifics of the switching activity. The following points describe the operation of the CSTA service response:

1.	Specific services may have an unconfirmed mode where responses to correct requests are not returned.

2.	The server shall check the correctness of the request (e.g. syntactical checks) before issuing the response. Incorrect requests shall result in an error response, even in the unconfirmed mode.

3.	If a response is sent before the action requested by the service is completed (i.e. the response is a service request acknowledgement) event reporting may be used to keep track of the subsequent server activity.

4.	The precise moment at which the response is generated in relation to the switching activity is implementation and service dependent.

i)	Some implementations may generate the response after checking the correctness of the request and at the point they initiate the request.

ii)	Other implementations may delay the response until the service has completed (or is guaranteed to complete). In this case, a failure of the switching request will be reflected in the response.

NOTE 1

Irrespective of implementation details, when an operation succeeds the same event reports are generated if the necessary monitoring has been established. As an example, a Held-Event (if selected) is always reported in addition to the response to a successful request (even in those implementations that delay the response until the Hold operation was complete). In a given context, and with appropriate monitoring in place, an operation generates the same set of event reports, whether it was invoked manually or with CSTA service requests from the computer.

## 5.4	Cross referencing of event reports

A computer application process may need to cross reference a CSTAEventReport to one of the following:

a)	a CSTA Object ID (Call ID or Device ID),

b)	an earlier Monitor request, or

c)	one of many Monitor requests (pertaining to the same CSTA Object).

For the above scenarios the necessary cross referencing function may be fulfilled by use of the parameter "MonitorCrossRefID". The content of MonitorCrossRefID depends upon the context and it may be one of the following: Call ID, StaticDevice ID, or another independently switch-managed static identifier. The independent identifier may have a unique correlation to either: one device, one call, or one monitor request.

The switching system limit on the number of monitor requests on one CSTA Object (Call or Device) is an implementation option. This Standard does not stipulate how many monitor requests on one object are to be supported by the switch. If using Static Device or Call identifiers the limit can only be one.

## 5.5	Handling of private data

If an entity receives the parameter CSTAPrivateData, and it can not recognize the information contained, the parameter shall be discarded, and the rest of the message shall be processed.

# 6	Interconnection service boundary

The protocol in this Standard is an OSI application layer protocol and uses the Remote Operations protocol defined in CCITT Rec. X.229. The Remote Operations protocol assumes certain services are provided by the underlying layers, and these services are also assumed by the protocol for CSTA.

# 7	Security

This protocol also provides a mechanism for secure transmission of CSTA PDUs as defined in this Standard. The parameters that constitute this secure PDU are imported from ECMA-138.

# Section 3 - CSTA Protocol

# 8	Association management

The protocol in this Standard operates in the context of an application association. It is assumed that such an application association exists via mechanisms not defined in this Standard.

# 9	Switching function services

This clause defines the protocol for the Switching Function services of CSTA, using ASN.1. Text descriptions of the CSTA services are provided in ECMA-179.

The encoding of the protocol is defined in CCITT Rec. X.209 " Specification of basic encoding rules for the abstract syntax notation".

NOTE 2

The range of services supported on a particular association is specified in the application context at association time.

## 9.1	Alternate call

CSTA-alternate-call

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) alternate-call( 1) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

ConnectionDetails FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) }

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

alternateCall	OPERATION

ARGUMENT	AlternateCallArgument

RESULT	AlternateCallResult

ERRORS	{universalFailure}

::= 1

AlternateCallArgument ::=

CHOICE

{callsInvolved	ConnectionDetails,

SEQUENCE

{callsInvolved	ConnectionDetails,

extensions	CSTACommonArguments	OPTIONAL} }

AlternateCallResult ::=

CHOICE

{extensions	CSTAPrivateData,

noData	NULL}

END  -- of CSTA-alternate-call

## 9.2	Answer call

CSTA-answer-call

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) answer-call( 2) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

answerCall OPERATION

ARGUMENT	AnswerCallArgument

RESULT	AnswerCallResult

ERRORS	{universalFailure}

::= 2

AnswerCallArgument ::=

CHOICE

{callToBeAnswered	ConnectionID,

SEQUENCE

{callToBeAnswered	ConnectionID,

extensions	CSTACommonArguments	OPTIONAL} }

AnswerCallResult ::=

CHOICE

{extensions	CSTAPrivateData,

noData	NULL}

END  -- of CSTA-answer-call

## 9.3	Call completion

CSTA-call-completion

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-completion( 3) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

FeatureInfo FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) }

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

callCompletion OPERATION

ARGUMENT	CallCompletionArgument

RESULT	CallCompletionResult

ERRORS	{universalFailure}

::= 3

CallCompletionArgument ::=

CHOICE

{featureInfo	FeatureInfo,

SEQUENCE

{featureInfo	FeatureInfo,

extensions	CSTACommonArguments	OPTIONAL} }

CallCompletionResult ::=

CHOICE

{extensions	CSTAPrivateData,

noData	NULL}

END  -- of CSTA-call-completion

## 9.4	Clear call

CSTA-clear-call

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) clear-call( 4) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

clearCall OPERATION

ARGUMENT	ClearCallArgument

RESULT	ClearCallResult

ERRORS	{universalFailure}

::= 4

ClearCallArgument ::=

CHOICE

{callToBeCleared	ConnectionID,

SEQUENCE

{callToBeCleared	ConnectionID,

extensions	CSTACommonArguments	OPTIONAL} }

ClearCallResult ::=

CHOICE

{extensions	CSTAPrivateData,

noData	NULL}

END  -- of CSTA-clear-call

## 9.5	Clear connection

CSTA-clear-connection

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) clear-connection( 5) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

clearConnection OPERATION

ARGUMENT	ClearConnectionArgument

RESULT	ClearConnectionResult

ERRORS	{universalFailure}

::= 5

ClearConnectionArgument ::=

CHOICE

{connectionToBeCleared	ConnectionID,

SEQUENCE

{connectionToBeCleared	ConnectionID,

extensions	CSTACommonArguments		OPTIONAL} }

ClearConnectionResult ::=

CHOICE

{extensions	CSTAPrivateData,

noData	NULL}

END  -- of CSTA-clear-connection

## 9.6	Conference call

CSTA-conference-call

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) conference-call( 6) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

ConnectionDetails FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) }

ConnectionList FROM CSTA-connection-states

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) connection-states( 125) }

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

conferenceCall OPERATION

ARGUMENT	ConferenceCallArgument

RESULT	ConferenceCallResult

ERRORS	{universalFailure}

::= 6

ConferenceCallArgument ::=

CHOICE

{callsInvolved	ConnectionDetails,

SEQUENCE

{callsInvolved	ConnectionDetails,

extensions	CSTACommonArguments	OPTIONAL} }

ConferenceCallResult ::=

SEQUENCE

{conferenceCall	ConnectionID,

connections	ConnectionList               	OPTIONAL,

extensions	CSTAPrivateData  		OPTIONAL}

END  -- of CSTA-conference-call

## 9.7	Consultation call

CSTA-consultation-call

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) consultation-call( 7) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

CalledDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

consultationCall OPERATION

ARGUMENT	ConsultationCallArgument

RESULT	ConsultationCallResult

ERRORS	{universalFailure}

::= 7

ConsultationCallArgument ::=

SEQUENCE

{existingCall	ConnectionID,

calledDirectoryNumber	CalledDeviceID,

extensions	CSTACommonArguments	OPTIONAL}

ConsultationCallResult ::=

CHOICE

{initiatedCall	ConnectionID,

SEQUENCE

{initiatedCall	ConnectionID,

extensions	CSTAPrivateData  	OPTIONAL} }

END  -- of CSTA-consultation-call

## 9.8	Divert call

CSTA-divert-call

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) divert-call( 8) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

DivertInfo FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

divertCall OPERATION

ARGUMENT	DivertCallArgument

RESULT	DivertCallResult

ERRORS	{universalFailure}

::= 8

DivertCallArgument ::=

CHOICE

{divertInfo	DivertInfo,

SEQUENCE

{divertInfo	DivertInfo,

extensions 	CSTACommonArguments 	OPTIONAL} }

DivertCallResult ::=

CHOICE

{extensions 	CSTAPrivateData,

noData	NULL}

END  -- of CSTA-divert-call

## 9.9	Hold call

CSTA-hold-call

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) hold-call( 9) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

ReserveConnection FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) }

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

holdCall OPERATION

ARGUMENT	HoldCallArgument

RESULT	HoldCallResult

ERRORS	{universalFailure}

::= 9

HoldCallArgument ::=

SEQUENCE

{callToBeHeld	ConnectionID,

connectionReservation	ReserveConnection  	DEFAULT FALSE,

extensions 	CSTACommonArguments	OPTIONAL}

HoldCallResult ::=

CHOICE

{extensions 	CSTAPrivateData,

noData	NULL}

END  -- of CSTA-hold-call

## 9.10	Make call

CSTA-make-call

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) make-call( 10) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

DeviceID, CalledDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

makeCall OPERATION

ARGUMENT	MakeCallArgument

RESULT	MakeCallResult

ERRORS	{universalFailure}

::= 10

MakeCallArgument ::=

SEQUENCE

{callingDevice	   DeviceID,

calledDirectoryNumber  CalledDeviceID,

extensions	   CSTACommonArguments	OPTIONAL}

MakeCallResult ::=

CHOICE

{initiatedCall	   ConnectionID,

SEQUENCE

{initiatedCall	   ConnectionID,

extensions	   CSTAPrivateData		OPTIONAL}}

END  -- of CSTA-make-call

## 9.11	Make Predictive call

CSTA-make-predictive-call

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) make-predictive-call( 11) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

AllocationState FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) }

DeviceID, CalledDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

makePredictiveCall  OPERATION

ARGUMENT	MakePredictiveCallArgument

RESULT	MakePredictiveCallResult

ERRORS	{universalFailure}

::= 11

MakePredictiveCallArgument ::=

SEQUENCE

{callingDevice	  DeviceID,

calledDirectoryNumber CalledDeviceID,

allocation	  AllocationState	             DEFAULT callDelivered,

extensions	  CSTACommonArguments	OPTIONAL}

MakePredictiveCallResult ::=

CHOICE

{initiatedCall	  ConnectionID,

SEQUENCE

{initiatedCall	  ConnectionID,

extensions	  CSTAPrivateData	           OPTIONAL} }

END  -- of CSTA-make-predictive-call

## 9.12	Query device

CSTA-query-device

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) query-device( 12) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

DeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

QueryDeviceFeature, QueryDeviceInformation FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

queryDevice OPERATION

ARGUMENT	QueryDeviceArgument

RESULT	QueryDeviceResult

ERRORS	{universalFailure}

::= 12

QueryDeviceArgument ::=

SEQUENCE

{device	DeviceID,

feature	QueryDeviceFeature,

extensions 	CSTACommonArguments	OPTIONAL}

QueryDeviceResult ::=

CHOICE

{deviceInformation	QueryDeviceInformation,

SEQUENCE

{deviceInformation	QueryDeviceInformation,

extensions	CSTAPrivateData		OPTIONAL} }

END  -- of CSTA-query-device

## 9.13	Reconnect call

CSTA-reconnect-call

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) reconnect-call( 13) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

ConnectionDetails FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) }

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

reconnectCall OPERATION

ARGUMENT	ReconnectCallArgument

RESULT	ReconnectCallResult

ERRORS	{universalFailure}

::= 13

ReconnectCallArgument ::=

CHOICE

{reconnectInfo	ConnectionDetails,

SEQUENCE

{reconnectInfo	ConnectionDetails,

extensions 	CSTACommonArguments 	OPTIONAL} }

ReconnectCallResult ::=

CHOICE

{extensions 	CSTAPrivateData,

noData	NULL}

END  -- of CSTA-reconnect-call

## 9.14	Retrieve call

CSTA-retrieve-call

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) retrieve-call( 14) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

retrieveCall OPERATION

ARGUMENT	RetrieveCallArgument

RESULT	RetrieveCallResult

ERRORS	{universalFailure}

::= 14

RetrieveCallArgument ::=

CHOICE

{callToBeRetrieved	ConnectionID,

SEQUENCE

{callToBeRetrieved	ConnectionID,

extensions	CSTACommonArguments	OPTIONAL} }

RetrieveCallResult ::=

CHOICE

{extensions	CSTAPrivateData,

noData	NULL}

END  -- of CSTA-retrieve-call

## 9.15	Set feature

CSTA-set-feature

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) set-feature( 15) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

DeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

SetDeviceFeature FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

setFeature OPERATION

ARGUMENT	SetFeatureArgument

RESULT	SetFeatureResult

ERRORS	{universalFailure}

::= 15

SetFeatureArgument ::=

SEQUENCE

{device	DeviceID,

feature	SetDeviceFeature,

extensions 	CSTACommonArguments	OPTIONAL}

SetFeatureResult ::=

CHOICE

{extensions 	CSTAPrivateData,

noData	NULL}

END  -- of CSTA-set-feature

## 9.16	Transfer call

CSTA-transfer-call

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) transfer-call( 16) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

ConnectionDetails FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) }

ConnectionID  FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

ConnectionList FROM CSTA-connection-states

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) connection-states( 125) }

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

transferCall OPERATION

ARGUMENT	TransferCallArgument

RESULT	TransferCallResult

ERRORS	{universalFailure}

::= 16

TransferCallArgument ::=

CHOICE

{transferInfo	ConnectionDetails,

SEQUENCE

{transferInfo	ConnectionDetails,

extensions 	CSTACommonArguments    OPTIONAL} }

TransferCallResult ::=

SEQUENCE

{transferredCall	ConnectionID		OPTIONAL,

connections	ConnectionList		OPTIONAL,

extensions 	CSTAPrivateData  		OPTIONAL}

END  -- of CSTA-transfer-call

# 10	Switching function events

This clause defines the protocol for the Event services of CSTA, using ASN.1. Text descriptions of the CSTA services are provided in ECMA-179.

NOTE 3

The range of services supported on a particular association is specified in the application context at association time.

The structure of an event is as described by the remote operation 'cSTAEventReport'. This is a ROSE class 5 operation, which has an argument containing the event type and other information associated with that event. The EVENT macro is used to define events in this standard. This macro provides a shorthand method to define the data types associated with a particular event.  Each use of the macro defines (after the EVENTINFO keyword) the data type that replaces the keyword 'ANY' in the cSTAEventReport operation for that particular event value.

CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions(21) }

DEFINITIONS    ::=

BEGIN

EXPORTS

EVENT;

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

MonitorCrossRefID FROM CSTA-status-reporting

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) status-reporting( 126) };

cSTAEventReport OPERATION

ARGUMENT	CSTAEventReportArgument

::= 21

CSTAEventReportArgument ::=

SEQUENCE

{crossRefIdentifier	MonitorCrossRefID,  -- allocated by switch

eventType		EventTypeID,

eventInfo		ANY DEFINED BY eventType,

extensions		CSTAPrivateData		OPTIONAL}

EventTypeID ::=  CHOICE { cSTAform       [0] IMPLICIT value(EVENT) }

EVENT   MACRO  ::=

BEGIN

TYPE  NOTATION	::= "EVENTINFO"  type  |   empty

VALUE  NOTATION	::= value(VALUE  INTEGER)

END  -- of EVENT macro

END  -- of CSTA-event-report-definitions

## 10.1	Call events

Each event contains a ConnectionID which identifies the object of interest. Other parameters may also be used to identify the relevant devices.

Events may result from a call interacting with switch features that had been previously set or invoked. These features and their settings may be reflected in an Event as a value of the EventCause parameter.

### 10.1.1	Call cleared

CSTA-call-cleared-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-cleared-event( 22) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

LocalConnectionState  FROM  CSTA-connection-states

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) connection-states( 125) }

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

EventCause FROM CSTA-event-causes

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-causes( 121) };

callCleared	EVENT

EVENTINFO 	CallClearedEventInfo

::= 1

CallClearedEventInfo ::=

SEQUENCE

{clearedCall		ConnectionID,

localConnectionInfo	LocalConnectionState	OPTIONAL,

cause		EventCause		OPTIONAL}

END  -- of CSTA-call-cleared-event

### 10.1.2	Conferenced

CSTA-conferenced-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) conferenced-event( 23) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

LocalConnectionState, ConnectionList FROM CSTA-connection-states

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) connection-states( 125) }

SubjectDeviceID    FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

EventCause FROM CSTA-event-causes

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-causes( 121) };

conferenced	EVENT

EVENTINFO 	ConferencedEventInfo

::= 2

ConferencedEventInfo ::=

SEQUENCE

{primaryOldCall	ConnectionID,

secondaryOldCall	ConnectionID		OPTIONAL,

confController	SubjectDeviceID,

addedParty		SubjectDeviceID,

conferenceConnections	ConnectionList		OPTIONAL,

localConnectionInfo	LocalConnectionState	OPTIONAL,

cause		EventCause		OPTIONAL}

END  -- of CSTA-conferenced-event

### 10.1.3	Connection cleared

CSTA-connection-cleared-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) connection-cleared-event( 24) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

LocalConnectionState FROM CSTA-connection-states

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) connection-states( 125) }

SubjectDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

EventCause FROM CSTA-event-causes

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-causes( 121) };

connectionCleared EVENT

EVENTINFO 	ConnectionClearedEventInfo

::= 3

ConnectionClearedEventInfo ::=

SEQUENCE

{droppedConnection	ConnectionID,

releasingDevice	SubjectDeviceID,

localConnectionInfo	LocalConnectionState	OPTIONAL,

cause		EventCause		OPTIONAL}

END  -- of CSTA-connection-cleared-event

### 10.1.4	Delivered

CSTA-delivered-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) delivered-event( 25) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

LocalConnectionState FROM CSTA-connection-states

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) connection-states( 125) }

SubjectDeviceID, CallingDeviceID, CalledDeviceID, RedirectionDeviceID

FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

EventCause FROM CSTA-event-causes

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-causes( 121) };

delivered	EVENT

EVENTINFO	DeliveredEventInfo

::= 4

DeliveredEventInfo ::=

SEQUENCE

{connection		ConnectionID,

alertingDevice	SubjectDeviceID,

callingDevice	CallingDeviceID,

calledDevice	CalledDeviceID,

lastRedirectionDevice	RedirectionDeviceID,

localConnectionInfo	LocalConnectionState	OPTIONAL,

cause		EventCause		OPTIONAL}

END  -- of CSTA-delivered-event

### 10.1.5	Diverted

CSTA-diverted-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) diverted-event( 26) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

LocalConnectionState FROM CSTA-connection-states

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) connection-states( 125) }

SubjectDeviceID, CalledDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

EventCause FROM CSTA-event-causes

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-causes( 121) };

diverted	EVENT

EVENTINFO	DivertedEventInfo

::= 5

DivertedEventInfo ::=

SEQUENCE

{connection		ConnectionID		OPTIONAL,

divertingDevice	SubjectDeviceID,

newDestination	CalledDeviceID,

localConnectionInfo	LocalConnectionState	OPTIONAL,

cause		EventCause		OPTIONAL}

END  -- of diverted-event

### 10.1.6	Established

CSTA--established-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) established-event( 27) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

LocalConnectionState FROM CSTA-connection-states

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) connection-states( 125) }

SubjectDeviceID, CalledDeviceID, CallingDeviceID, RedirectionDeviceID

FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

EventCause FROM CSTA-event-causes

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-causes( 121) };

established	EVENT

EVENTINFO	EstablishedEventInfo

::= 6

EstablishedEventInfo ::=

SEQUENCE

{establishedConnection	ConnectionID,

answeringDevice	SubjectDeviceID,

callingDevice	CallingDeviceID,

calledDevice	CalledDeviceID,

lastRedirectionDevice	RedirectionDeviceID,

localConnectionInfo	LocalConnectionState	OPTIONAL,

cause		EventCause		OPTIONAL}

END  -- of CSTA-established-event

### 10.1.7	Failed

CSTA-failed-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) failed-event( 28) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

LocalConnectionState FROM CSTA-connection-states

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) connection-states( 125) }

SubjectDeviceID, CalledDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

EventCause FROM CSTA-event-causes

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-causes( 121) };

failed		EVENT

EVENTINFO	FailedEventInfo

::= 7

FailedEventInfo ::=

SEQUENCE

{failedConnection	ConnectionID,

failingDevice	SubjectDeviceID,

calledDevice	CalledDeviceID,

localConnectionInfo	LocalConnectionState	OPTIONAL,

cause		EventCause		OPTIONAL}

END  -- of CSTA-failed-event

### 10.1.8	Held

CSTA-held-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) held-event( 29) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

LocalConnectionState FROM CSTA-connection-states

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) connection-states( 125) }

SubjectDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

EventCause FROM CSTA-event-causes

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-causes( 121) };

held		EVENT

EVENTINFO	HeldEventInfo

::= 8

HeldEventInfo ::=

SEQUENCE

{heldConnection	ConnectionID,

holdingDevice	SubjectDeviceID,

localConnectionInfo	LocalConnectionState	OPTIONAL,

cause		EventCause		OPTIONAL}

END  -- of CSTA-held-event

### 10.1.9	Network reached

CSTA-network-reached-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) network-reached-event( 30) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

LocalConnectionState FROM CSTA-connection-states

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) connection-states( 125) }

SubjectDeviceID, CalledDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

EventCause FROM CSTA-event-causes

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-causes( 121) };

networkReached	EVENT

EVENTINFO	NetworkReachedEventInfo

::= 9

NetworkReachedEventInfo ::=

SEQUENCE

{connection		ConnectionID,

trunkUsed		SubjectDeviceID,

calledDevice	CalledDeviceID,

localConnectionInfo	LocalConnectionState	OPTIONAL,

cause		EventCause		OPTIONAL}

END  -- of CSTA-network-reached-event

### 10.1.10	Originated

CSTA-originated-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) originated-event( 31) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

LocalConnectionState FROM CSTA-connection-states

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) connection-states( 125) }

SubjectDeviceID, CalledDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

EventCause FROM CSTA-event-causes

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-causes( 121) };

originated	EVENT

EVENTINFO	OriginatedEventInfo

::= 10

OriginatedEventInfo ::=

SEQUENCE

{originatedConnection	ConnectionID,

callingDevice	SubjectDeviceID,

calledDevice	CalledDeviceID,

localConnectionInfo	LocalConnectionState	OPTIONAL,

cause		EventCause		OPTIONAL}

END  -- of CSTA-originated-event

### 10.1.11	Queued

CSTA-queued-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) queued-event( 32) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

LocalConnectionState FROM CSTA-connection-states

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) connection-states( 125) }

CalledDeviceID, CallingDeviceID, RedirectionDeviceID, SubjectDeviceID

FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

NoOfCallsInQueue FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) }

EventCause FROM CSTA-event-causes

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-causes( 121) };

queued	EVENT

EVENTINFO	QueuedEventInfo

::= 11

QueuedEventInfo ::=

SEQUENCE

{queuedConnection	ConnectionID,

queue		SubjectDeviceID,

callingDevice	CallingDeviceID,

calledDevice	CalledDeviceID,

lastRedirectionDevice	RedirectionDeviceID,

numberedQueued	NoOfCallsInQueue		OPTIONAL,

localConnectionInfo	LocalConnectionState	OPTIONAL,

cause		EventCause		OPTIONAL}

END  -- of CSTA-queued-event

### 10.1.12	Retrieved

CSTA-retrieved-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) retrieved-event( 33) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

LocalConnectionState FROM CSTA-connection-states

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) connection-states( 125) }

SubjectDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

EventCause FROM CSTA-event-causes

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-causes( 121) };

retrieved	EVENT

EVENTINFO	RetrievedEventInfo

::= 12

RetrievedEventInfo ::=

SEQUENCE

{retrievedConnection	ConnectionID,

retrievingDevice	SubjectDeviceID,

localConnectionInfo	LocalConnectionState	OPTIONAL,

cause		EventCause		OPTIONAL}

END  -- of CSTA-retrieved-event

### 10.1.13	Service initiated

CSTA-service-initiated-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) service-initiated-event( 34) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

LocalConnectionState FROM CSTA-connection-states

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) connection-states( 125) }

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

EventCause FROM CSTA-event-causes

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-causes( 121) };

serviceInitiated	EVENT

EVENTINFO	ServiceInitiatedEventInfo

::= 13

ServiceInitiatedEventInfo ::=

SEQUENCE

{initiatedConnection	ConnectionID,

localConnectionInfo	LocalConnectionState	OPTIONAL,

cause		EventCause		OPTIONAL}

END  -- of CSTA-service-initiated-event

### 10.1.14	Transferred

CSTA-transferred-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) transferred-event( 35) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

LocalConnectionState, ConnectionList FROM CSTA-connection-states

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) connection-states( 125) }

SubjectDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

EventCause FROM CSTA-event-causes

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-causes( 121) };

transferred	EVENT

EVENTINFO	TransferedEventInfo

::= 14

TransferedEventInfo ::=

SEQUENCE

{primaryOldCall	ConnectionID,

secondaryOldCall	ConnectionID		OPTIONAL,

transferringDevice	SubjectDeviceID,

transferredDevice	SubjectDeviceID,

transferredConnections	ConnectionList		OPTIONAL,

localConnectionInfo	LocalConnectionState	OPTIONAL,

cause		EventCause		OPTIONAL}

END  -- of CSTA-transferred-event

## 10.2	Feature events

### 10.2.1	Call information

CSTA-call-information-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-information-event( 41) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

SubjectDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

AccountInfo, AuthCode FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) }

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) };

callInformation	EVENT

EVENTINFO	CallInformationEventInfo

::= 101

CallInformationEventInfo ::=

SEQUENCE

{connection		ConnectionID,

device		SubjectDeviceID,

accountInfo		[10] IMPLICIT AccountInfo	OPTIONAL,

authorisationCode	[11] IMPLICIT AuthCode	OPTIONAL}

END  -- of CSTA-call-information-event

### 10.2.2	Do not disturb

CSTA-do-not-disturb-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) do-not-disturb-event( 42) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

SubjectDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) };

doNotDisturb	EVENT

EVENTINFO	DoNotDisturbEventInfo

::= 102

DoNotDisturbEventInfo ::=

SEQUENCE

{device		SubjectDeviceID,

doNotDisturbOn	BOOLEAN}

END  -- of CSTA-do-not-disturb-event

### 10.2.3	Forwarding

CSTA-forwarding-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) forwarding-event( 43) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

ForwardParameter FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) }

SubjectDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) };

forwarding	EVENT

EVENTINFO	ForwardingEventInfo

::= 103

ForwardingEventInfo ::=

SEQUENCE

{device		SubjectDeviceID,

forwardingInformation	ForwardParameter}

END  -- of CSTA-forwarding-event

### 10.2.4	Message waiting

CSTA-message-waiting-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) message-waiting-event( 44) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

CalledDeviceID, SubjectDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) };

messageWaiting	EVENT

EVENTINFO	MessageWaitingEventInfo

::= 104

MessageWaitingEventInfo ::=

SEQUENCE

{deviceForMessage	CalledDeviceID,

invokingDevice	SubjectDeviceID,

messageWaitingOn	BOOLEAN}

END  -- of CSTA-message-waiting-event

## 10.3	Agent state events

### 10.3.1	Logged on

CSTA-logged-on-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) logged-on-event( 51) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

SubjectDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

AgentID, AgentGroup, AgentPassword FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) };

loggedOn	EVENT

EVENTINFO	LoggedOnEventInfo

::= 201

LoggedOnEventInfo ::=

SEQUENCE

{agentDevice	SubjectDeviceID,

agentID		[10] IMPLICIT AgentID	OPTIONAL,

agentGroup		AgentGroup		OPTIONAL,

password		[11] IMPLICIT AgentPassword	OPTIONAL}

END  -- of CSTA-logged-on-event

### 10.3.2	Logged off

CSTA-logged-off-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) logged-off-event( 52) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

SubjectDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

AgentID, AgentGroup FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) };

loggedOff	EVENT

EVENTINFO	LoggedOffEventInfo

::= 202

LoggedOffEventInfo ::=

SEQUENCE

{agentDevice	SubjectDeviceID,

agentID		[10] IMPLICIT AgentID  OPTIONAL,

agentGroup		AgentGroup	    OPTIONAL}

END  -- of CSTA-logged-off-event

### 10.3.3	Not ready

CSTA-not-ready-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) not-ready-event( 53) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

SubjectDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123)}

AgentID FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) };

notReady     	EVENT

EVENTINFO	NotReadyEventInfo

::= 203

NotReadyEventInfo ::=

SEQUENCE

{agentDevice	SubjectDeviceID,

agentID		[10] IMPLICIT AgentID	OPTIONAL}

END  -- of CSTA-not-ready-event

### 10.3.4	Ready

CSTA-ready-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) ready-event( 54) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

SubjectDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

AgentID FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) };

ready  	EVENT

EVENTINFO	ReadyEventInfo

::= 204

ReadyEventInfo ::=

SEQUENCE

{agentDevice	SubjectDeviceID,

agentID		[10] IMPLICIT AgentID	OPTIONAL}

END  -- of CSTA-ready-event

### 10.3.5	Work not ready

CSTA-work-not-ready-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) work-not-ready-event( 55) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

SubjectDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

AgentID FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) };

workNotReady  	EVENT

EVENTINFO	WorkNotReadyEventInfo

::= 205

WorkNotReadyEventInfo ::=

SEQUENCE

{agentDevice	SubjectDeviceID,

agentID		[10] IMPLICIT AgentID	OPTIONAL}

END  -- of CSTA-work-not-ready-event

### 10.3.6	Work ready

CSTA-work-ready-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) work-ready-event( 56) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

SubjectDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

AgentID FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) };

workReady     	EVENT

EVENTINFO	WorkReadyEventInfo

::= 206

WorkReadyEventInfo ::=

SEQUENCE

{agentDevice	SubjectDeviceID,

agentID		[10] IMPLICIT AgentID	OPTIONAL}

END  -- of CSTA-work-ready-event

## 10.4	Maintenance events

### 10.4.1	Back in service

CSTA-back-in-service-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) back-in-service-event( 61) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

DeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

EventCause FROM CSTA-event-causes

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-causes( 121) };

backInService	EVENT

EVENTINFO	BackInServiceEventInfo

::= 301

BackInServiceEventInfo ::=

SEQUENCE

{device		DeviceID,

cause		EventCause	OPTIONAL}

END  -- of CSTA-back-in-service-event

### 10.4.2	Out of service event

CSTA-out-of-service-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) out-of-service-event( 62) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) }

-- Data Types --

DeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

EventCause FROM CSTA-event-causes

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-causes( 121) };

outOfService	EVENT

EVENTINFO	OutOfServiceEventInfo

::= 302

OutOfServiceEventInfo ::=

SEQUENCE

{device		DeviceID,

cause		EventCause	OPTIONAL}

END  -- of CSTA-out-of-service-event

## 10.5	Private events

CSTA-private-event

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) private-event( 71) }

DEFINITIONS    ::=

BEGIN

IMPORTS

EVENT FROM CSTA-event-report-definitions

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-report-definitions( 21) };

private		EVENT

EVENTINFO	PrivateEventInfo

::= 401

PrivateEventInfo ::= NULL

-- The actual encoding of the private event is added here, replacing NULL with another

-- valid ASN.1 type.

END	-- of CSTA-private-event

# 11	Computing function services

This clause defines the protocol for the Computing function services of CSTA, using ASN.1. Text descriptions of the CSTA services are provided in ECMA-179.

NOTE 4

The range of services supported on a particular association is specified in the application context at association time.

## 11.1	Route request

CSTA-route-request

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) route-request( 81) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

CalledDeviceID, CallingDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

SelectValue, PriorityValue, SetUpValues, RoutingCrossRefID FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) }

CSTACommonArguments FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

routeRequest OPERATION

ARGUMENT	RouteRequestArgument

ERRORS	{universalFailure}

::= 31

RouteRequestArgument ::=

SEQUENCE

{crossRefIdentifier		RoutingCrossRefID,

currentRoute		CalledDeviceID,

callingDevice		CallingDeviceID		OPTIONAL,

routedCall		ConnectionID		OPTIONAL,

routeSelAlgorithm		SelectValue		OPTIONAL,

priority		PriorityValue		OPTIONAL,

setupInformation		SetUpValues		OPTIONAL,

extensions		CSTACommonArguments	OPTIONAL}

END  -- of CSTA-route-request

## 11.2	Re-route request

CSTA-re-route-request

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) re-route-request( 82) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

RoutingCrossRefID FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) }

CSTACommonArguments FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

reRouteRequest OPERATION

ARGUMENT	ReRouteRequestArgument

ERRORS	{universalFailure}

::= 32

ReRouteRequestArgument ::=

SEQUENCE

{crossRefIdentifier	RoutingCrossRefID,

extensions 	CSTACommonArguments		OPTIONAL}

END  -- of CSTA-re-route-request

## 11.3	Route select request

CSTA-Route-select-request

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) route-select-request( 83) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

CalledDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

RouteUsedFlag, RetryValue, SetUpValues, RoutingCrossRefID FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) }

CSTACommonArguments FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

routeSelectRequest	OPERATION

ARGUMENT	RouteSelectRequestArgument

ERRORS	{universalFailure}

::= 33

RouteSelectRequestArgument ::=

SEQUENCE

{crossRefIdentifier		RoutingCrossRefID,

routeSelected		CalledDeviceID,

remainRetry		RetryValue		OPTIONAL,

setupInformation		SetUpValues		OPTIONAL,

routeUsedReq		RouteUsedFlag		OPTIONAL,

extensions 		CSTACommonArguments	OPTIONAL}

END  -- of CSTA-route-select-request

## 11.4	Route used request

CSTA-route-used-request

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) route-used-request( 84) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

CallingDeviceID, CalledDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

DomainValue, RoutingCrossRefID FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) }

CSTACommonArguments FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

routeUsedRequest	OPERATION

ARGUMENT	RouteUsedRequestArgument

ERRORS	{universalFailure}

::= 34

RouteUsedRequestArgument ::=

SEQUENCE

{crossRefIdentifier		RoutingCrossRefID,

routeUsed		CalledDeviceID,

callingDevice		CallingDeviceID		OPTIONAL,

domain		DomainValue		OPTIONAL,

extensions 		CSTACommonArguments	OPTIONAL}

END  -- of CSTA-route-used-request

## 11.5	Route end request

CSTA-route-end-request

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) route-end-request( 85) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

RoutingCrossRefID  FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) }

CSTACommonArguments FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure, UniversalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

routeEndRequest	OPERATION

ARGUMENT	RouteEndRequestArgument

ERRORS	{universalFailure}

::= 35

RouteEndRequestArgument ::=

SEQUENCE

{crossRefIdentifier		RoutingCrossRefID,

errorValue		UniversalFailure		OPTIONAL,

extensions 		CSTACommonArguments	OPTIONAL}

END  -- of CSTA-route-end-request

# 12	Bidirectional services

This clause defines the protocol for the Bidirectional services of CSTA, using ASN.1. Text descriptions of the CSTA services are provided in ECMA-179.

NOTE 5

The range of services supported on a particular association is specified in the application context at association time.

## 12.1	Escape service

CSTA-escape-service

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) CSTA-escape-service( 91) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1(1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

escapeService	OPERATION

ARGUMENT	EscapeServiceArgument

RESULT	EscapeServiceResult

ERRORS	{universalFailure}

::=  51

EscapeServiceArgument ::=

extensions 	CSTACommonArguments

EscapeServiceResult ::=

CHOICE

{extensions 	CSTAPrivateData,

noData	NULL}

END  -- of CSTA-escape-service

## 12.2	System status

CSTA-system-status

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) system-status( 92) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

SystemStatus FROM CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

systemStatus	OPERATION

ARGUMENT	SystemStatusArgument

RESULT	SystemStatusResult

ERRORS	{universalFailure}

::= 52

SystemStatusArgument ::=

CHOICE

{systemStatus	SystemStatus,

SEQUENCE

{systemStatus	SystemStatus,

extensions	CSTACommonArguments	OPTIONAL} }

SystemStatusResult ::=

CHOICE

{extensions	CSTAPrivateData,

noData	NULL}

END  -- of CSTA-system-status

# 13	Status reporting services

This clause defines the protocol for the Status reporting services of CSTA, using ASN.1. Text descriptions of the CSTA services are provided in ECMA-179.

NOTE 6

The range of services supported on a particular association is specified in the application context at association time.

## 13.1	Monitor start

CSTA-monitor-start

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) monitor-start( 101) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

MonitorObject, MonitorFilter, MonitorType, MonitorCrossRefID FROM CSTA-status-reporting

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) status-reporting( 126) }

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

monitorStart	OPERATION

ARGUMENT	MonitorStartArgument

RESULT	MonitorStartResult

ERRORS	{universalFailure}

::= 71

MonitorStartArgument ::=

SEQUENCE

{monitorObject	MonitorObject,

monitorFilter	MonitorFilter		OPTIONAL,

monitorType	MonitorType		OPTIONAL,

extensions	CSTACommonArguments	OPTIONAL}

MonitorStartResult ::=

SEQUENCE

{crossRefIdentifier	MonitorCrossRefID,

monitorFilter	MonitorFilter		OPTIONAL,

extensions	CSTAPrivateData		OPTIONAL}

END  -- of CSTA-monitor-start

## 13.2	Change monitor filter

CSTA-change-monitor-filter

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1 ( 1) change-monitor-filter( 102) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

MonitorFilter, MonitorCrossRefID FROM CSTA-status-reporting

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) status-reporting( 126) }

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

changeMonitorFilter	OPERATION

ARGUMENT	ChangeMonitorFilterArgument

RESULT	ChangeMonitorFilterResult

ERRORS	{universalFailure}

::= 72

ChangeMonitorFilterArgument ::=

SEQUENCE

{monitorCrossRefID	MonitorCrossRefID,

filterlist	MonitorFilter,

extensions	CSTACommonArguments	OPTIONAL}

ChangeMonitorFilterResult ::=

CHOICE

{filterList	[0] IMPLICIT MonitorFilter,

SEQUENCE

{filterList	MonitorFilter	OPTIONAL,

extensions	CSTAPrivateData	OPTIONAL} }

END  -- of CSTA-change-monitor-filter

## 13.3	Monitor stop

CSTA-monitor-stop

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) monitor-stop( 103) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

MonitorCrossRefID FROM CSTA-status-reporting

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) status-reporting( 126) }

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

monitorStop	OPERATION

ARGUMENT	MonitorStopArgument

RESULT	MonitorStopResult

ERRORS 	{universalFailure}

::= 73

MonitorStopArgument ::=

CHOICE

{crossRefIdentifier         MonitorCrossRefID,

SEQUENCE

{crossRefIdentifier    MonitorCrossRefID,

extensions               CSTACommonArguments    	OPTIONAL} }

MonitorStopResult ::=

CHOICE

{extensions	CSTAPrivateData,

noData	NULL}

END  -- of CSTA-monitor-stop

## 13.4	Snapshot device

CSTA-snapshot-device

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) snapshot-device( 104) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

SnapshotDeviceData FROM CSTA-status-reporting

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) status-reporting( 126) }

DeviceID FROM Device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

snapshotDevice	OPERATION

ARGUMENT	SnapshotDeviceArgument

RESULT	SnapshotDeviceResult

ERRORS	{universalFailure}

::= 74

SnapshotDeviceArgument ::=

CHOICE

{snapshotObject	DeviceID,

SEQUENCE

{snapshotObject	DeviceID,

extensions	CSTACommonArguments	OPTIONAL} }

SnapshotDeviceResult ::=

CHOICE

{snapshotData	SnapshotDeviceData,

SEQUENCE

{snapshotData	SnapshotDeviceData,

extensions	CSTAPrivateData		OPTIONAL} }

END  -- of CSTA-snapshot-device

## 13.5	Snapshot call

CSTA-snapshot-call

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) snapshot-call( 105) }

DEFINITIONS    ::=

BEGIN

IMPORTS

OPERATION, ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

-- Data Types --

SnapshotCallData FROM CSTA-status-reporting

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) status-reporting( 126) }

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

universalFailure FROM CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) };

snapshotCall	OPERATION

ARGUMENT	SnapshotCallArgument

RESULT	SnapshotCallResult

ERRORS	{universalFailure}

::= 75

SnapshotCallArgument ::=

CHOICE

{snapshotObject	ConnectionID,

SEQUENCE

{snapshotObject	ConnectionID,

extensions 	CSTACommonArguments	OPTIONAL} }

SnapshotCallResult ::=

CHOICE

{snapshotData	SnapshotCallData,

SEQUENCE

{snapshotData	SnapshotCallData,

extensions 	CSTAPrivateData		OPTIONAL} }

END  -- of CSTA-snapshot-call

# 14	Switching function errors

CSTA-error-definition

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) error-definition( 120) }

DEFINITIONS    ::=

BEGIN

EXPORTS UniversalFailure, universalFailure;

IMPORTS ERROR FROM Remote-Operations-Notation

{ joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

CSTAPrivateData FROM CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) };

universalFailure	ERROR

PARAMETER	UniversalFailure

::= 1

UniversalFailure ::= CHOICE

{ operationalErrors 	[1] IMPLICIT Operations,

stateErrors		[2] IMPLICIT StateIncompatibility,

systemResourceErrors	[3] IMPLICIT SystemResourceAvailability,

subscribedResourceAvailabilityErrors	[4] IMPLICIT SubscribedResourceAvailability,

performanceErrors	[5] IMPLICIT PerformanceManagement,

securityErrors		[6] IMPLICIT SecurityError,

unspecifiedErrors	[7] IMPLICIT NULL,

nonStandardErrors	     CSTAPrivateData}

Operations ::= ENUMERATED

{	generic		  (1),

requestIncompatibleWithObject	  (2),

valueOutOfRange		  (3),

objectNotKnown		  (4),

invalidCallingDevice		  (5),

invalidCalledDevice		  (6),

invalidForwardingDestination		  (7),

privilegeViolationOnSpecifiedDevice	  (8),

privilegeViolationOnCalledDevice	  (9),

privilegeViolationOnCallingDevice	(10),

invalidCSTACallIdentifier		(11),

invalidCSTADeviceIdentifier		(12),

invalidCSTAConnectionIdentifier	(13),

invalidDestination		(14),

invalidFeature		(15),

invalidAllocationState		(16),

invalidCrossRefID		(17),

invalidObjectType		(18),

securityViolation		(19)   }

StateIncompatibility ::= ENUMERATED

{	generic		  (1),

invalidObjectState		  (2),

invalidConnectionID		  (3),

noActiveCall		  (4),

noHeldCall		  (5),

noCallToClear		  (6),

noConnectionToClear		  (7),

noCallToAnswer		  (8),

noCallToComplete		  (9) }

SystemResourceAvailability ::= ENUMERATED

{	generic		  (1),

serviceBusy		  (2),

resourceBusy		  (3),

resourceOutOfService		  (4),

networkBusy		  (5),

networkOutOfService		  (6),

overallMonitorLimitExceeded		  (7),

conferenceMemberLimitExceeded 	  (8)   }

SubscribedResourceAvailability ::= ENUMERATED

{	generic		  (1),

objectMonitorLimitExceeded		  (2),

externalTrunkLimitExceeded		  (3),

outstandingRequestLimitExceeded 	  (4)   }

PerformanceManagement ::= ENUMERATED

{	generic		  (1),

performanceLimitExceeded		  (2)   }

SecurityError ::= ENUMERATED

{	unspecified		                             (0),

sequenceNumberViolated	                             (1),

timeStampViolated		                             (2),

pACViolated		                             (3),

sealViolated		                             (4)   }

END  -- of CSTA-error-definition

# 15	Switching event cause values

CSTA-event-causes

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) event-causes( 121) }

DEFINITIONS ::=

BEGIN

EXPORTS   EventCause;

EventCause ::= ENUMERATED	-- a general list of cause codes

{ 	activeMonitor	(1),

alternate	(2),

busy	(3),

callBack	(4),

callCancelled	(5),

callForwardAlways	(6),

callForwardBusy	(7),

callForwardNoAnswer	(8),

callForward	(9),

callNotAnswered	(10),

callPickup	(11),

campOn	(12),

destNotObtainable	(13),

doNotDisturb	(14),

incompatibleDestination	(15),

invalidAccountCode	(16),

keyConference	(17),

lockout	(18),

maintenance	(19),

networkCongestion	(20),

networkNotObtainable	(21),

newCall	(22),

noAvailableAgents	(23),

override	(24),

park	(25),

overflow	(26),

recall	(27),

redirected	(28),

reorderTone	(29),

resourcesNotAvailable	(30),

silentMonitor	(31),

transfer	(32),

trunksBusy	(33),

voiceUnitInitiator	(34)     }

END  -- of event-cause-definitions

# 16	CSTA data types

The major parameters have been assigned distinct application tags to facilitate parsing.  The data is defined in logical groups in ascending order of application tag. Application tags used are:

.	APPLICATION  1 - 5 :		Device identifiers

.	APPLICATION  11 - 14 :	Connection identifiers and local connection states

.	APPLICATION  21 - 24 :	Status reporting

.	APPLICATION  29 :		CSTAPrivateData

.	APPLICATION  30 :		CSTACommonArguments

## 16.1	Switching function objects

CSTA-switching-function-objects

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) switching-function-objects( 122) }

DEFINITIONS    ::=

BEGIN

EXPORTS  CSTAObject;

IMPORTS

DeviceID  FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

ConnectionID FROM  CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) };

CSTAObject	::= CHOICE

{device	DeviceID,

call	ConnectionID}

END  -- of CSTA-switching-function-objects

## 16.2	Device identifiers

CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

DEFINITIONS    ::=

BEGIN

EXPORTS

DeviceID, NumberDigits, ExtendedDeviceID, CallingDeviceID, CalledDeviceID,

SubjectDeviceID, RedirectionDeviceID;

DeviceID	::= CHOICE

{dialingNumber	[0] IMPLICIT	NumberDigits,

deviceNumber	[1] IMPLICIT	DeviceNumber }

-- NumberDigits is a string of digits that represents a number (address) that

-- the switch can route a call on. It can be dialled by a user  (i.e from a telephone

-- keypad) to make a call.  DeviceNumber is not a routing address and cannot be dialed

-- from a keypad. It is allocated by the switch to reference a device.

NumberDigits	::=  IA5String

DeviceNumber	::=  INTEGER

ExtendedDeviceID	::= CHOICE

{deviceIdentifier			DeviceID,

implicitPublic	[2]	IMPLICIT	NumberDigits,

explicitPublic	[3]		PublicTON,

implicitPrivate	[4]	IMPLICIT	NumberDigits,

explicitPrivate	[5]		PrivateTON,

other	[6]	IMPLICIT	OtherPlan }

CallingDeviceID	::= [APPLICATION 1] CHOICE

{deviceIdentifier      	ExtendedDeviceID,

notKnown	[7]	IMPLICIT NULL,

notRequired	[8]	IMPLICIT NULL }

CalledDeviceID 	::= [APPLICATION 2]  CHOICE

{deviceIdentifier	ExtendedDeviceID,

notKnown	[7]	IMPLICIT NULL,

notRequired	[8]	IMPLICIT NULL }

SubjectDeviceID 	::= [APPLICATION 3]  CHOICE

{deviceIdentifier	ExtendedDeviceID,

notKnown	[7]	IMPLICIT NULL,

notRequired	[8]	IMPLICIT NULL }

RedirectionDeviceID	::= [APPLICATION 4] CHOICE

{numberdialed	ExtendedDeviceID,

notKnown	[7]	IMPLICIT	NULL,

notRequired	[8]	IMPLICIT	NULL }

-- SubjectDeviceID is used in some event reports to specify which device the report

-- refers to. If the SubjectDeviceID has had a monitor invoked then this data is not

-- required and so the implicit NULL encoding for notRequired is returned.

-- RedirectionDeviceID is used in Events as the lastRedirectionDevice.

-- CalledDeviceID is used in Events to specify the number dialled.

PublicTON ::= CHOICE

{	unknown		[0]	IMPLICIT IA5String,

international		[1]	IMPLICIT IA5String,

national		[2]	IMPLICIT IA5String,

networkspecific	[3]	IMPLICIT IA5String,

subscriber		[4]	IMPLICIT IA5String,

abbreviated		[5]	IMPLICIT IA5String }

-- the public type of numbers  are derived from CCITT E.164

PrivateTON ::= CHOICE

{	unknown		[0]	IMPLICIT IA5String,

level3RegionalNumber	[1]	IMPLICIT IA5String,

level2RegionalNumber	[2]	IMPLICIT IA5String,

level1RegionalNumber	[3]	IMPLICIT IA5String,

pTNSpecificNumber	[4]	IMPLICIT IA5String,

localNumber		[5]	IMPLICIT IA5String,

abbreviated		[6]	IMPLICIT IA5String }

-- the private type of numbers are derived from ECMA-155

OtherPlan ::=  OCTET STRING    -- allows future expansion to cover other numbering

-- plans (such as X.121 etc.)

END  -- of CSTA-device-identifiers

## 16.3	Call and connection identifiers

CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

DEFINITIONS    ::=

BEGIN

EXPORTS

ConnectionID;

IMPORTS

DeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) };

ConnectionID	::=[APPLICATION 11]	IMPLICIT SEQUENCE

{	call	                              [2]	IMPLICIT OCTET STRING  OPTIONAL,

device     CHOICE

{staticID	DeviceID,

dynamicID             [3]	IMPLICIT OCTET STRING }  OPTIONAL}

END  -- of CSTA-call-connection-identifiers

## 16.4	Connection states

CSTA-connection-states

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) connection-states( 125) }

DEFINITIONS    ::=

BEGIN

EXPORTS

ConnectionList, ConnectionIDList, LocalConnectionState;

IMPORTS

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

DeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) };

ConnectionIDList	::= [APPLICATION 12] IMPLICIT SEQUENCE OF ConnectionID

CallInfo  ::= [APPLICATION 13] IMPLICIT SEQUENCE OF SEQUENCE

{endpoint	ConnectionID,

staticEndpoint	DeviceID  	OPTIONAL }

ConnectionList  ::= CHOICE

{connections	ConnectionIDList,

callinformation	CallInfo }

LocalConnectionState  ::= [APPLICATION 14] IMPLICIT ENUMERATED

{	null	(0),

initiate	(1),

alerting	(2),

connect	(3),

hold	(4),

queued	(5),

fail	(6) }

END  -- CSTA-connection-states

## 16.5	Status reporting

CSTA-status-reporting

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) status-reporting( 126) }

DEFINITIONS    ::=

BEGIN

EXPORTS

MonitorObject, MonitorCrossRefID, MonitorFilter, MonitorType, SnapshotCallData,

SnapshotDeviceData;

IMPORTS

DeviceID, SubjectDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) }

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

LocalConnectionState FROM CSTA-connection-states

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) connection-states( 125) }

CSTAObject FROM CSTA-switching-function-objects

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) switching-function-objects( 122) };

MonitorObject ::= CSTAObject

MonitorCrossRefID ::= [APPLICATION 21] IMPLICIT OCTET STRING

MonitorFilter	::=  SEQUENCE	-- default is no filter (i.e. all events)

{	call	[0] IMPLICIT CallFilter	DEFAULT {}

feature	[1] IMPLICIT FeatureFilter	DEFAULT {}

agent	[2] IMPLICIT AgentFilter	DEFAULT {}

maintenance	[3] IMPLICIT MaintenanceFilter	DEFAULT {}

private	[4] IMPLICIT BOOLEAN	DEFAULT FALSE  }

-- setting the relevant bit requests the filter for the appropriate events

CallFilter	::=  BIT STRING

{	callCleared		  (0),

conferenced		  (1),

connectionCleared	  (2),

delivered		  (3),

diverted		  (4),

established		  (5),

failed		  (6),

held		  (7),

networkReached	  (8),

originated		  (9),

queued		(10),

retrieved		(11),

serviceInitiated	(12),

transferrd		(13)    }

FeatureFilter	::=  BIT STRING

{	callInformation	(0),

doNotDisturb		(1),

forwarding		(2),

messageWaiting	(3)    }

AgentFilter	::=  BIT STRING

{	loggedOn		(0),

loggedOff		(1),

notReady		(2),

ready		(3),

workNotReady	(4),

workReady		(5)    }

MaintenanceFilter	::=  BIT STRING

{	backInService	(0),

outOfService		(1)    }

MonitorType ::=  ENUMERATED

{	call             (0),

device        (1)    }

SnapshotDeviceData ::= [APPLICATION 22] IMPLICIT SEQUENCE OF 					  		    SnapshotDeviceResponseInfo

SnapshotDeviceResponseInfo ::=  SEQUENCE

{callIdentifier		ConnectionID,

localCallState	CallState    }

SnapshotCallData ::= [APPLICATION 23] IMPLICIT SEQUENCE OF 					  		    SnapshotCallResponseInfo

SnapshotCallResponseInfo ::=  SEQUENCE

{deviceOnCall	SubjectDeviceID,

callIdentifier		ConnectionID,

localConnectionState	LocalConnectionState   OPTIONAL }

CallState   ::= CHOICE

{ compound	[0] IMPLICIT CompoundCallState,

simple	[1] IMPLICIT SimpleCallState,

unknown	[2] IMPLICIT NULL              }

-- unknown is returned by server if no other CallState can be supplied --

CompoundCallState   ::= SEQUENCE OF LocalConnectionState

SimpleCallState	::=  ENUMERATED

{callNull         	(0),     -- '00'H   - null-null

callPending      	(1),     -- '01'H   - null-initiate

callOriginated   	(3),     -- '03'H   - null-connect

callDelivered               	(35),    -- '23'H   - alerting-connect

callDeliveredHeld	(36),    -- '24'H   - alerting-held

callReceived	(50),    -- '32'H   - connect-alerting

callEstablished        	(51),    -- '33'H   - connect-connect

callEstablishedHeld    	(52),    -- '34'H   - connected-held

callReceivedOnHold     	(66),    -- '42'H   - held-alerting

callEstablishedOnHold  	(67),    -- '43'H   - held-connect

callQueued		(83),    -- '53'H   - queued-connect

callQueuedHeld	(84),    -- '54'H   - queued-held

callFailed     	(99),    -- '63'H   - failed-connect

callFailedHeld	(100)}   -- '64'H   - failed-held

-- This represents the main call states in a simplified encoding.  The semantics

-- are identical to the sequence of connection states but they are represented by

-- an item from an enumerated list.

END  -- of CSTA-status-reporting

## 16.6	Device and feature types and other parameters

CSTA-device-feature-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-feature-types( 127) }

DEFINITIONS    ::=

BEGIN

EXPORTS

AccountInfo, AgentID, AgentGroup, AgentPassword, AgentParameter, AgentState,

AllocationState, AuthCode, ConnectionDetails, DeviceClass, DeviceInfo, DeviceType,

DivertInfo, FeatureInfo, ListForwardParameters, LoggedOnInfo, LoggedOffInfo,

ForwardParameter, ForwardingType, NoOfCallsInQueue, QueryDeviceFeature,

QueryDeviceInformation, ReserveConnection, SetDeviceFeature, SystemStatus, SelectValue,

PriorityValue, SetUpValues, RetryValue, RouteUsedFlag, DomainValue, RoutingCrossRefID;

IMPORTS

ConnectionID FROM CSTA-call-connection-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) call-connection-identifiers( 124) }

DeviceID, NumberDigits, CallDeviceID FROM CSTA-device-identifiers

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) device-identifiers( 123) };

AccountInfo	::= OCTET STRING	-- used in CallInformation event

AgentID 	::= OCTET STRING

AgentGroup 	::= DeviceID

AgentPassword 	::= OCTET STRING

AgentParameter 	::= CHOICE

{	loggedIn		[0]	IMPLICIT LoggedOnInfo,

loggedOut		[1]	IMPLICIT LoggedOffInfo,

notReady		[2]	IMPLICIT NULL,

ready		[3]	IMPLICIT NULL,

workNotReady	[4]	IMPLICIT NULL,

workReady		[5]	IMPLICIT NULL  }

AgentState	::= ENUMERATED

{	notReady		(0),

null		(1),

ready		(2),

workNotReady	(3),

workReady		(4) }

AllocationState	::= ENUMERATED

{	callDelivered      (0),

callEstablished   (1) }

-- used in MakePredictiveCall to indicate when call should be allocated to the device

AuthCode	::= OCTET STRING   -- used in Information Events

ConnectionDetails ::= CHOICE

{	 heldCall	[0] IMPLICIT ConnectionID,

activeCall	[1] IMPLICIT ConnectionID,

bothCalls	[2] IMPLICIT SEQUENCE

{heldCall    ConnectionID,

activeCall  ConnectionID} }

DeviceClass ::= BIT STRING

{	voice	(0),

data	(1),

image	(2),

other	(3) }

DeviceInfo	:: =  SEQUENCE

{	deviceID            DeviceID 	OPTIONAL,

deviceType        DeviceType	OPTIONAL,

deviceClass       DeviceClass	OPTIONAL}

DeviceType ::= ENUMERATED

{	station		    (0),

line		    (1),

button		    (2),

aCD		    (3),

trunk		    (4),

operator		    (5),

station-group		  (16),

line-group		  (17),

button-group		  (18),

aCD-group		  (19),

trunk-group		  (20),

operator-group	  (21),

other		(255)   }

DivertInfo	::= CHOICE	--used by Divert Call service

{  deflect	[0] IMPLICIT SEQUENCE

{ callToBeDiverted    ConnectionID,

newDestination      CalledDeviceID }

pickup	[1] IMPLICIT SEQUENCE

{ callToBePickedUp    ConnectionID,

requestingDevice    DeviceID }

group	[2] DeviceID      }

FeatureInfo	::= CHOICE	--used by Call Completion service

{	campon	       [0] IMPLICIT	ConnectionID,

callback	       [1] IMPLICIT	ConnectionID,

intrude	       [2] IMPLICIT	ConnectionID }

ListForwardParameters  ::= SEQUENCE OF SEQUENCE

{	forwardingType	ForwardingType,

forwardDN		NumberDigits }

LoggedOnInfo	::= SEQUENCE

{	agentID	      [10] IMPLICIT	AgentID	OPTIONAL,

password	      [11] IMPLICIT	AgentPassword	OPTIONAL,

group	              	AgentGroup	OPTIONAL  }

LoggedOffInfo	::= SEQUENCE

{	agentID	      [10] IMPLICIT	AgentID	OPTIONAL,

group	              	AgentGroup	OPTIONAL  }

ForwardParameter    ::= SEQUENCE

{	forwardingType	ForwardingType,

forwardDN		NumberDigits	OPTIONAL    }

ForwardingType	::= ENUMERATED

{	forwardImmediateOn	(0),

forwardImmediateOff	(1),

forwardBusyOn	(2),

forwardBusyOff	(3),

forwardNoAnsOn	(4),

forwardNoAnsOff	(5),

forwardBusyIntOn	(6),

forwardBusyIntOff	(7),

forwardBusyExtOn	(8),

forwardBusyExtOff	(9),

forwardNoAnsIntOn	(10),

forwardNoAnsIntOf	(11),

forwardNoAnsExtOn	(12),

forwardNoAnsExtOff	(13) }

NoOfCallsInQueue	::= INTEGER	-- used in Call Queued Event

QueryDeviceFeature  ::=ENUMERATED	-- used by Query Feature service request

{	msgWaitingOn   	(0),

doNotDisturbOn 	(1),

forward	   	(2),

lastDialedNumber    	(3),

deviceInfo	   	(4),

agentState	   	(5) }

QueryDeviceInformation   ::= CHOICE

{	msgWaitingOn	[0]	IMPLICIT BOOLEAN,

doNotDisturbOn	[1]	IMPLICIT BOOLEAN,

forward		[2]	IMPLICIT ListForwardParameters,

lastDialed		[3]	IMPLICIT NumberDigits,

deviceInfo		[4]	IMPLICIT DeviceInfo,

agentState		[5]	IMPLICIT AgentState  }

ReserveConnection	::= BOOLEAN	-- used with Hold service to reserve ISDN

-- connection

SetDeviceFeature	::= CHOICE	-- used by SetFeature service request

{	msgWaitingOn	[0]	IMPLICIT BOOLEAN,

doNotDisturbOn	[1]	IMPLICIT BOOLEAN,

forward		[2]	IMPLICIT ForwardParameter,

aRequestedAgentState	[3]	AgentParameter }

SystemStatus ::= ENUMERATED

{	initializing		(0),

enabled		(1),

normal		(2),

messagesLost	(3),

disabled		(4),

overloadImminent	(5),

overloadReached	(6),

overloadRelieved	(7) }

SelectValue ::= ENUMERATED

{	normal		(0),

leastCost		(1),

emergency		(2),

aCD		(3),

userDefined		(4) }

PriorityValue ::= BOOLEAN	-- TRUE means priority call

SetUpValues ::= OCTET STRING	-- Contains Q.931 Setup message

RetryValue ::= CHOICE	-- used in RouteSelect Request service

{	noListAvailable	[0] IMPLICIT BOOLEAN,

noCountAvailable	[1] IMPLICIT BOOLEAN,

retryCount		[2] IMPLICIT INTEGER  }

RouteUsedFlag ::= BOOLEAN	-- TRUE means RouteUsed Request service 				-- requested

DomainValue ::= BOOLEAN	-- TRUE means CSTA subdomain destination

RoutingCrossRefID ::= [APPLICATION 24] IMPLICIT OCTET STRING

END  -- of CSTA-device-feature-types

## 16.7	Security service

CSTA-security

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) security( 128) }

DEFINITIONS    ::=

BEGIN

EXPORTS

CSTASecurityData;

IMPORTS

PrivilegeAttributeCertificate FROM Security-Information

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) desd( 138) securityData( 1) };

Seal ::= SEQUENCE

{	algorithmIdentifier	OBJECT IDENTIFIER,

keyIdentifier		OCTET STRING,

seal	  	OCTET STRING  }

CSTASecurityData	::= SEQUENCE

{ messageSequenceNumber	INTEGER		OPTIONAL,

timeStamp		UTCTime		OPTIONAL,

privilegeAttributeCertificate	PrivilegeAttributeCertificate								OPTIONAL,

seal		Seal		OPTIONAL }

END  -- of CSTA-security

## 16.8	Common extensions

CSTA-extension-types

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) extension-types( 129) }

DEFINITIONS    ::=

BEGIN

EXPORTS

CSTACommonArguments, CSTAPrivateData;

IMPORTS

CSTASecurityData FROM CSTA-security

{ iso( 1) identified-organization( 3) icd-ecma( 0012)

standard( 0) csta( 180) version1( 1) security( 128) };

CSTACommonArguments ::= [APPLICATION 30] IMPLICIT SEQUENCE

{	security		[0] IMPLICIT CSTASecurityData	OPTIONAL

privateData		[1] IMPLICIT SEQUENCE OF CSTAPrivateData

OPTIONAL }

CSTAPrivateData    ::= [APPLICATION 29] IMPLICIT SEQUENCE

{	manufacturer	OBJECT IDENTIFIER,

ANY DEFINED BY	manufacturer}

-- Manufacturer specific (or standard) extensions shall be uniquely identified using

-- Object Identifiers issued by ECMA according to ISO 6523.

END  -- of CSTA-extension-types